Preventive Maintenance Checklist

English

Trendsetter/Achieve 400/800/News Platesetters

SA/CL/AU/AL

Customer Name:	PT. Satyamitra Kemas Lestari

Address:	Jl. Raya Serang Km 25.6 Tigaraksa - Tangerang 15720

Device serial number:	TJ1782

Kodak contact:	PT. Microlux Indonesia

Date:	25 November 2025

Copyright

© Kodak, 2022. All rights reserved.

Kodak and Creo are trademarks of Kodak.

http://graphics.kodak.com.

Internal 732-00266A-EN Rev D
Revised 2025-11-25

Preventive Maintenance Checklist

This Preventive Maintenance Checklist is for Trendsetter/Achieve 400/800/News platesetters:

- Series II (TMCE, serial numbers TMxxx and NMxxx)

- Series III (GMCE, serial numbers TGxxx and NGxxx)

- Series IV (GMCE, serial numbers TTxxxx)

- Series V (EMCE, serial numbers TJxxxx)

with automation options:

- Semi-Automatic (SA aka BASE)

- Continues Load (CL)

- Auto-Unload (AU)

- Auto-Loader (AL)

For SCU and MCU maintenance, refer to Answer ID 31844

The objective of preventive maintenance (PM) is to maximize uptime for Trendsetter/Achieve 400/800/News platesetters. 
Specific automation items are shown in parenthesis. Only mark a Pass for checklist items that apply to the platesetter on which you are working.

Refer to the detailed PM procedure for more information about each step, Answer ID 75627.

- Safety and General operation

|  | Safety and General Operation | Pass | Needs attention | Comments |
| --- | --- | --- | --- | --- |
|  | Operator feedback |  |  | Summarize customer experiences below: |
| Plate sometimes rejected during loading process, based on log had a debris nozzle flow error (43006 & 43010). Do nozzle cleaning to overcome the error. |  |  |  |  |
|  | Image reference plate |  |  |  |
|  | Environmental conditions |  |  | Little bit dusty |
|  | Doors and safety switches |  |  |  |
|  | E-Stop switches test |  |  |  |
|  | Compressed air – filters replaced by customer, bowls checked, no leaks, no oil, no water |  |  | No regulator filter, recommend to use air filter regulator |
|  | Air pressure at 90 psi and stable during operation |  |  | 87 psi and stable |
|  | Solenoid valve manifolds – no leaks |  |  | No leaks but very dusty |
|  | Intake air filter checked. Replaced by customer |  |  | Replaced |
|  | Intake fan(s) – air flow and noise check |  |  |  |
|  | Heat exchanger – no leaks |  |  |  |
|  | Coolant level OK |  |  | Using non Kodak coolant |
|  | Check bypassed sensors |  |  |  |
|  | Record the image stats |  |  | 61715 |
| - Submit a job from the workstation. (normal operation with doors closed) - Watch and listen. Note irregularities for further investigation later. |  |  |  |  |
|  | Plate loading onto drum |  |  |  |
|  | Carriage operation, noise |  |  |  |
|  | Plate unload from drum |  |  |  |
|  | Picker movement smooth (AL) |  |  |  |

- Imaging

|  | Imaging | Pass | Needs attention | Comments |
| --- | --- | --- | --- | --- |
|  | Carriage |  |  |  |
|  | Carriage rails or linear bearings lubricated |  |  |  |
|  | Carriage lead screw cleaned and lubricated |  |  |  |
|  | [Optional] Debris Removal |  |  |  |
|  | Verify need for UDRC via media technote. Disable UDRC when not required. |  |  | Need UDRC |
|  | Filter in good condition |  |  |  |
|  | Hose in good condition |  |  |  |
|  | Nozzle sensor hose clean |  |  |  |
|  | [Optional] External Chiller |  |  |  |
|  | External chiller PM; refer to Neslab/Polyscience for TH1.7 (Answer ID 14044) Neslab TF1400 for TH7 (Answer ID 43697) |  |  |  |
|  | Image Quality |  |  |  |
|  | Final window clean |  |  |  |
|  | Head position (sum/error) |  |  |  |
|  | Laser power and drum speed within spec |  |  | Using Fuji LH-PK plate |
|  | Image quality good |  |  |  |

- Power Box and Engine

|  | Device engine | Pass | Needs attention | Comments |
| --- | --- | --- | --- | --- |
|  | Power Box |  |  |  |
|  | AC power and grounding |  |  |  |
|  | Cooling fans operation |  |  |  |
|  | Engine |  |  |  |
|  | Load ramp cylinder mounting secure |  |  |  |
|  | Load ramp cylinder ball lock pins secure |  |  |  |
|  | Drum Roller clean and in good condition |  |  | Dusty, need to do cleaning |
|  | Drum Roller spins smoothly |  |  |  |
|  | Drum Roller moves evenly |  |  |  |
|  | LEC moves evenly |  |  | LEC away actuator possibly leak, recommend to be replaced |
|  | LECs in good condition, springs operate smoothly |  |  |  |
|  | TEC magnets clean and not broken |  |  |  |
|  | TEC lock/unlock smooth |  |  |  |
|  | Edge detect strip scratch free. Color black if needed. |  |  | Scratch free but dusty, need to do cleaning |

- Devices with Autoloader/Unload Table

|  | CL/AU/AL devices only | Pass | Needs attention | Comments |  |
| --- | --- | --- | --- | --- | --- |
|  | AL Picker |  |  |  |  |
|  | Bay door lock operation |  |  |  |  |
|  | Picker guide cables and pulleys inspection |  |  |  |  |
|  | Picker guide cables tension |  |  |  |  |
|  | Picker up/down cylinders no leaks/loose jam nuts (AL) |  |  |  |  |
|  | Check E-clips and cotter pins |  |  |  |  |
|  | Wrist cylinders, no leaks/loose jam nuts |  |  |  |  |
|  | Inspect Mylar strip |  |  |  |  |
|  | Picker vacuum cup check: replace if necessary |  |  |  |  |
|  | Picker head distance to load table |  |  |  |  |
|  | ‘Picker out’ timing after last separation tooth |  |  |  |  |
|  | Picker actuator timings |  |  | Picker up Picker in Picker wrist in Picker wrist out Picker out Picker down Separator in Separator out |  |
|  | Plate to drum contact during load, ~1 inch above LECs. (AL) |  |  |  |  |
|  | Plate drop height 2 – 3 mm from the loadtable to reg pins (AL) |  |  |  |  |
|  | Separator arms operation |  |  |  |  |
|  | [Optional] TS NEWS Dual-Bay. Refer to Answer ID 12430 |  |  |  |  |
|  | CL Load Table |  |  |  |  |
|  | Loadtable vacuum cup check: replace if necessary |  |  |  |  |
|  | Plate drop height ~4 mm |  |  |  |  |
|  | Unload Table |  |  |  |  |
|  | Unload table surface clean |  |  |  |  |
|  | Unload table, no popped-up buttons/rivets |  |  |  |  |
|  | Table cylinders, no leaks, clevis and pin replaced (if worn) and greased |  |  |  |  |
|  | [Optional] Rotator cylinder, no leaks, proper rotation |  |  |  |  |
|  | Cup cylinders, smooth operation correct heights |  |  |  |  |
|  | Unload vacuum cup inspection, replace if necessary |  |  |  |  |
|  | Inspect plate pusher operation and pressure |  |  |  |  |
|  | Unload table optical sensor clean |  |  |  |  |

- Workstation

|  | Workstation and Controlled Release | Pass | Needs attention | Comments |
| --- | --- | --- | --- | --- |
|  | Cables and connections in good condition |  |  |  |
|  | Automatic Backups enabled in Service Tools |  |  |  |
|  | Test operation (compare reference plate and send production jobs from Print Console) |  |  |  |
|  | Hard drive image capture recommended |  |  |  |

- Finishing up

- Clean off any finger prints that you may have left on the panels during work on the device.

- Speak with an operator or maintenance person to point out any problems or lack of operator maintenance that you noticed.

- Ensure that the customer understands their responsibility in ordering filters and to replace them when needed.

- Ensure that any applicable parts are returned to Kodak.

Note any:

- Safety features disabled or bypassed

- Operating environment issues

- Operator training required

PM completion

| PM is complete | Next PM: |  |
| --- | --- | --- |
| PM is partially complete to step | Date to complete PM: | 25 November 2025 |
|  | Reason: |  |
|  | Laser on Time Count: | 7039 hrs |